# CHE251 TERM PROJECT PROPOSAL

Proposed by: Prakhar Gupta

Team members: Arun Chauhan, Prince Meena, Anubhav Jha, Aman Agrawal, Rajinder Kumar, Arjit Agrawal, Tripti

Date: 5th September 2025

## Title: Process Simulation and Optimization of Acetone Production from Isopropanol

### Background and Motivation

Acetone (CH3COCH3) is one of the most important solvents in the industry. It is used for pharmaceuticals, coatings, textiles and much more. The shift from older batch methods to continuous catalytic dehydrogenation of isopropanol (IPA) has made production more scalable and efficient. However, as with many equilibrium-limited reactions, the process faces challenges: incomplete single-pass conversion, energy demand for endothermic reactions, and management of Inert in the recycle loop.

Our motivation is to study and optimize the process through simulation and schematic design, which is essential to minimize costs, reducing energy usage and increasing acetone yield (desired product). This will be done through the teachings of CHE251.

### 2. The Working Process and Its Evolution

#### a. Chemical Reaction:

The main endothermic reaction in the reactor is:

CH3CH(OH)CH3(g) → CH3COCH3(g) + H2(g)

-Catalyst: Zinc oxide, copper, or supported noble metals.

- Operating conditions: 200–400 °C, moderate pressure.

- Single-pass IPA conversion: ~85–90%.

b. Separation & Purification:

- Cooling & phase separation: Hot reactor effluent is cooled, releasing hydrogen.

- Distillation: Separates acetone from unreacted IPA.

- Recycle loop: Unreacted IPA is compressed and recycled.

#### c. Recycle & Purge:

- Recycle: Maximizes utilization of feed, pushing overall conversion toward 100%.

- Purge: Prevents accumulation of inert gases (e.g., nitrogen, impurities) that could dilute reactants

### Process Summary and Problems

The base process involves

IPA feed → fixed-bed catalytic reactor → separation → purification.

Problems:

- Incomplete single-pass conversion.

- Accumulation of inert in recycle loop.

- Significant heat duty (endothermic reaction)

### Objectives

- Developing a base case flowsheet (DWSIM).

- Performing full material and energy balancing (this topic will be taught after midsemester).

- Implementing a recycle loop for unreacted IPA.

- Incorporating a purge stream and analysing the effect on conversion and efficiency.

- Compare performance with vs. without recycle/purge.

### Workplan

Phase 1: Simulate base-case (reactor only).

Phase 2: Add separation unit, obtain base separation performance.

Phase 3: Introduce recycle loop, converge flowsheet.

Phase 4: Add purge stream, analyse inert accumulation and effects.

Phase 5: Compare material balances, efficiency, and yield improvements.

Phase 6: Will work on additional deliverables (if time permits).

### Key References

- https://www.researchgate.net/publication/369022935_Production_of_Acetone_by_Dehydrogenation_of_Isopropyl_alcohol

- -Himmelblau, D. M. & Riggs, J. B. Basic Principles and Calculations in Chemical Engineering (9th ed.).

- Pearson, 2024. Felder, R. M. & Rousseau, R. W. Elementary -Principles of Chemical Processes (2nd ed.). Wiley, 1986. Turton, R., Bailie, R. C., Whiting, W. B., & Shaeiwitz, J. A. Analysis, Synthesis, and Design of Chemical Processes (5th ed.). Pearson, 2018

### 7. Minimum Deliverables

- Converged simulation file prepared in DWSIM.

- Process flowsheet diagram including reactor, separation, recycle, and purge streams.

- Material balance table summarizing stream compositions and flowrates.

- Energy balance table for key process units.

- Comparative analysis of base-case and optimized process performance.

- Additional Deliverables

- Economic analysis of recycle vs. compressor/energy.

- Heat integration: Using hot effluent to preheat IPA feed.

- Environmental impact: Reduction in waste and energy footprint.

### Timeline

- September-

Team formation, topic finalization, Simulating base case and preparing mid semester report

- October- Adding separation unit, implementing recycle and purge loop and comparison

- November- Preparing final report, creating 10 min video presentation,

Additional deliverables (if time permits)